Step 1: Understand and Define the Problem (Analyse)

The goal is to design a combinational logic circuit that aperates an alarm when:

- The car ignition is ON

- Either the passenger or the driver seats are occuoied

- The corresponding seatbelt is not fastened

This means:

- If the driver is in the seat and the seatbelt is not fastened

- If the passeneger is in the seat and the seatbelt is not fastened

- If the car is on

Step 2: Organise and Describe the Data

| Data Symbol | Type | Description | Active Logic |
| --- | --- | --- | --- |
| DRIV | INPUT | Driver seat occupied | 1 or HIGH |
| PASS | INPUT | Passeneger seat occupied | 1 or HIGH |
| IGN | INPUT | Ignition switch is ON | 1 or HIGH |
| BELTD | INPUT | Driver seat belt fastened | 1 or HIGH |
| BELTP | INPUT | Passenger seat belt fastened | 1 or HIGH |
| ALARM | OUTPUT | Car alarm activiated | 0 or LOW |

Step 3: Plan the Solution (Design the Algorithm)

3.1 Algorithm (in plain English)

- Monitor ignition status: IGN must be HIGH

- Check if driver is stated and drivers belt is not fastened

  - DRIV = 1 AND BELTD = 0

- Check if passenger is seated and passengers belt is not fastened

  - PASS = 1 AND BELTP = 0

- If either condition above is TRUE and IGN = 1, the ALARM activates (LOW or 0)

- Otherwise, alarm remains HIGH or 1 (off)

3.2 Truth Table

| DRIV | PASS | BELTD | BELTP | IGN | ALARM |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

3.3 Boolean Expressions (SOP)

| DRIV | PASS | BELTD | BELTP | IGN | ALARM |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |

- DRIV, PASS, BELTD, BELTP, IGN

- DRIV, PASS, BELTD, BELTP, IGN

- DRIV, PASS, BELTD, BELTP, IGN

- DRIV, PASS, BELTD, BELTP, IGN

- DRIV, PASS, BELTD, BELTP, IGN

ALARM =  DRIV, PASS, BELTD, BELTP, IGN + DRIV, PASS, BELTD, BELTP, IGN + DRIV, PASS, BELTD, BELTP, IGN + DRIV, PASS, BELTD, BELTP, IGN + DRIV, PASS, BELTD, BELTP, IGN

3.4 Flowchart

3.5 Pseudocode

START

GET DRIV, PASS, BELTD, BELTP, IGN

IF DRIV.  PASS.  BELTD.  BELTP.  IGN + DRIV.  PASS.  BELTD.  BELTP.  IGN + DRIV.  PASS. BELTD.  BELTP.  IGN + DRIV. PASS.  BELTD. BELTP. IGN + DRIV. PASS. BELTD.  BELTP. IGN

THEN

SET ALARM to ON (0)

ELSE

SET ALARM to OFF (1)

END

Step 4: Test and Refine the Solution (Debug and Verify)

Test Data:

| DRIV | PASS | BELTD | BELTP | IGN | Expected Output ALARM | Actual Output | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 |  |  |
| 1 | 0 | 0 | 0 | 1 | 0 |  |  |
| 1 | 0 | 0 | 1 | 1 | 0 |  |  |
| 1 | 0 | 1 | 0 | 1 | 1 |  |  |
| 1 | 0 | 1 | 1 | 1 | 1 |  |  |
| 1 | 1 | 0 | 0 | 1 | 0 |  |  |
| 1 | 1 | 0 | 1 | 1 | 0 |  |  |
| 1 | 1 | 1 | 0 | 1 | 0 |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 |  |  |